RAMCO VIDYA MANDIR SR. SEC. SCHOOL, THAMARAIKULAM Subject: Physics JEE EXAM

CLASS : 12-A; TIME :. EXAM NO :; MARKS : 40.

SECTION – Multiple Choice Questions SECTION (25 MARKS)

1. In the Ampere-Maxwell law, the displacement current term was introduced to account for:
(a) Current flow through insulators
(b) Time-varying electric field between capacitor plates
(c) Magnetic monopoles
(d) Conduction current in wires

2. A parallel plate capacitor with circular plates of radius 8 cm is being charged. If the displacement current is 0.2 A, the rate of change of electric flux between the plates is:
(a) 0.2/ε₀ Wb/s
(b) 0.2ε₀ Wb/s
(c) 0.2 Wb/s
(d) 0.2μ₀ Wb/s

3. The electromagnetic wave equation relates wave vector k and angular frequency ω as:
(a) ω = k/c
(b) ω = ck
(c) ω² = ck
(d) ω = c/k

4. Which scientist first experimentally demonstrated electromagnetic waves in laboratory?
(a) Maxwell
(b) Faraday
(c) Hertz
(d) Marconi

5. The greenhouse effect on Earth is primarily maintained by which electromagnetic waves?
(a) Ultraviolet
(b) Visible light
(c) Infrared
(d) Microwaves

6. For electromagnetic waves propagating in a medium with permittivity ε and permeability μ, the velocity is:
(a) √(μ/ε)
(b) 1/√(με)
(c) √(με)
(d) 1/(μ+ε)

7. In a plane electromagnetic wave, if E = 9 V/m at a point, the magnetic field B at that point is:
(a) 3 × 10⁻⁸ T
(b) 2.7 × 10⁻⁹ T
(c) 3 × 10⁻⁹ T
(d) 27 T

8. The frequency range of FM radio band is:
(a) 530 kHz to 1710 kHz
(b) 88 MHz to 108 MHz
(c) 54 MHz to 890 MHz
(d) 500 kHz to 1000 MHz

9. X-rays are commonly produced by:
(a) Oscillating electric dipoles
(b) Hot bodies
(c) Bombarding metal targets with high energy electrons
(d) Radioactive decay

10. The electromagnetic waves that can be focused into very narrow beams for LASIK surgery are:
(a) Infrared
(b) Visible light
(c) Ultraviolet
(d) X-rays

11. Which layer in atmosphere protects us from harmful UV radiation?
(a) Troposphere
(b) Stratosphere
(c) Ozone layer
(d) Ionosphere

12. The wavelength range of visible light is approximately:
(a) 400 nm to 700 nm
(b) 10 nm to 400 nm
(c) 700 nm to 1 mm
(d) 1 mm to 10 cm

13. Jagdish Chandra Bose succeeded in producing electromagnetic waves of wavelength:
(a) 25 m to 5 m
(b) 25 cm to 5 cm
(c) 25 mm to 5 mm
(d) 25 μm to 5 μm

14. In electromagnetic waves, the electric field E and magnetic field B are:
(a) Parallel to each other and perpendicular to direction of propagation
(b) Perpendicular to each other and parallel to direction of propagation
(c) Perpendicular to each other and perpendicular to direction of propagation
(d) At 45° to each other

15. The source of electromagnetic waves is:
(a) Stationary charges
(b) Charges in uniform motion
(c) Accelerated charges
(d) Magnetic monopoles

16. Microwave ovens work on the principle of:
(a) Heating by conduction
(b) Resonant frequency matching with water molecules
(c) Chemical reactions
(d) Nuclear fission

17. The electromagnetic waves used in cellular phone communication are in:
(a) VHF band
(b) UHF band
(c) HF band
(d) LF band

18. For a wave with frequency ν and wavelength λ propagating in vacuum:
(a) νλ = c²
(b) ν/λ = c
(c) νλ = c
(d) ν²λ = c

19. Gamma rays are produced by:
(a) Klystron valves
(b) Hot bodies
(c) Nuclear reactions
(d) Electric arcs

20. The displacement current density is given by:
(a) ε₀(dE/dt)
(b) μ₀(dB/dt)
(c) ε₀E
(d) μ₀H

21. Infrared detectors in Earth satellites are used for:
(a) Communication
(b) Observing crop growth
(c) GPS navigation
(d) Weather prediction only

22. The speed of electromagnetic waves in vacuum is:
(a) Dependent on frequency
(b) Dependent on wavelength
(c) Independent of frequency and wavelength
(d) Dependent on amplitude

23. Which electromagnetic waves are sometimes called 'heat waves'?
(a) Radio waves
(b) Microwaves
(c) Infrared waves
(d) Ultraviolet waves

24. The refractive index of a medium depends on:
(a) Only electric properties
(b) Only magnetic properties
(c) Both electric and magnetic properties
(d) Neither electric nor magnetic properties

25. TV transmission uses electromagnetic waves in the range:
(a) 54 MHz to 890 MHz
(b) 530 kHz to 1710 kHz
(c) 88 MHz to 108 MHz
(d) 1 GHz to 10 GHz

SECTION – Numerical Answer Type SECTION (15 MARKS)

26. A parallel plate capacitor with circular plates of radius 15 cm is being charged at a constant rate. If the displacement current is 0.45 A, calculate the rate of change of electric field between the plates in V/(m·s). (Use ε₀ = 8.85 × 10⁻¹² F/m)

27. An electromagnetic wave has a magnetic field amplitude of 6 × 10⁻⁷ T. Calculate the electric field amplitude in V/m.

28. A radio station broadcasts at 98.5 MHz. Calculate the wavelength of the electromagnetic waves in meters (rounded to 2 decimal places).

29. The electric field in an electromagnetic wave is given by E = 250 sin(1.5 × 10⁷z - 4.5 × 10¹⁵t) V/m. Calculate the wavelength in nanometers (rounded to nearest integer).

30. A parallel plate capacitor has circular plates of radius 4 cm separated by 2 mm. If it is connected to a 50 Hz AC source and the displacement current amplitude is 8.85 × 10⁻⁸ A, calculate the peak voltage across the capacitor in volts.

31. An electromagnetic wave has frequency 2.4 GHz. Calculate its wave number k in rad/m (rounded to 2 decimal places).

32. If the electric field amplitude of an electromagnetic wave is 360 N/C, calculate the intensity of the wave in W/m² (use ε₀ = 8.85 × 10⁻¹² F/m).

33. A microwave oven operates at 2.45 GHz. Calculate the energy of one photon in eV (use h = 4.14 × 10⁻¹⁵ eV·s, rounded to 6 decimal places).

34. The magnetic field in a plane electromagnetic wave is By = 8 × 10⁻⁶ sin(2 × 10³x + 6 × 10¹¹t) T. Calculate the frequency in GHz (rounded to 1 decimal place).

35. An X-ray has wavelength 0.5 nm. Calculate its frequency in Hz (express as 10ⁿ Hz, give value of n).

36. A capacitor with plate area 0.02 m² and separation 1 mm is being charged. If the electric field between plates changes at 10¹² V/(m·s), calculate the displacement current in amperes (use ε₀ = 8.85 × 10⁻¹² F/m).

37. An electromagnetic wave in a medium has velocity 2 × 10⁸ m/s. Calculate the refractive index of the medium (rounded to 2 decimal places).

38. The wavelength of yellow light is 580 nm. Calculate its frequency in THz (rounded to nearest integer).

39. A gamma ray has energy 1.24 MeV. Calculate its wavelength in meters (use hc = 1.24 × 10⁻⁶ eV·m, express as 10⁻ⁿ m, give value of n).

40. An electromagnetic wave propagating along x-axis has E = 45ĵ V/m at a point. Calculate the magnitude of Poynting vector in W/m² (use μ₀ = 4π × 10⁻⁷ H/m).